Email proposals in Microsoft Word format to info@ncwm.com by August 15.

Each Regional Association will hold hearings on proposals in the fall.  See meeting dates at www.ncwm.com/meetings.  If any region deems that the item has merit, the region will forward the item to NCWM for national consideration.  For more information on the Form 15 process, visit www.ncwm.com/standards-dev.

| GENERAL INFORMATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Proposal to: |  |  |  |  |  |  |  |
| __ Laws & Regulations _X_ Specifications & Tolerances __ Professional Development __ Board of Directors __ NTEP Committee |  |  |  |  |  |  |  |
| 2. Submitter’s Name: |  |  |  |  | 3. Date: |  |  |
| Michael Keilty / Brent Price |  |  |  |  | September 16, 2025 |  |  |
| 4. Submitter’s Organization: |  |  | 5. Address: |  |  |  |  |
| Measuring Sector |  |  | 2441 Arapaho Road |  |  |  |  |
| 6. City: |  | 7. State: |  |  |  | 8. Zip Code: | 9. Country: |
| Estes Park |  | Colorado |  |  |  | 80517 | USA |
| 10. Phone Number: | 11. Fax Number: |  |  | 12. Email Address: |  |  |  |
| 317-701-0823 |  |  |  | michael.keilty@endress.com |  |  |  |
| PROPOSAL INFORMATION |  |  |  |  |  |  |  |
| 13. Purpose: Concise statement as to the intent or purpose of this proposal, such as problem being fixed. (Do not include justification here.) |  |  |  |  |  |  |  |
| The Method of Sealing Category 3 language is different in several of the Measuring Code sections. In 2021, the HB 44 Code Section 3.40 EVFS Table S.3.3 and in 2022, the HB 44 Code Section 3.30 LMD Table S.2.2 Methods of Sealing, Category 3 were amended. The purpose of this proposal is to amend the same section found in the other HB 44 Device 3.XX Code Sections with the same language. |  |  |  |  |  |  |  |
| 14. Document to be Amended: |  |  |  |  |  |  |  |
| X Handbook 44 Handbook 130 Handbook 133 __ NCWM Guidance Document __ NCWM Bylaws __ NTEP Administrative Policy |  |  |  |  |  |  |  |
| 15. Cite portion to be Amended: Submit a separate Form 15 for each code, model law or regulation to be amended. |  |  |  |  |  |  |  |
| Section: 3.30 LMD Table S.2.2, 3.31 VTM Table S.2.2., 3.32 LPG&AALM Table S.2.2., 3.34 CLM Table S.2.5., 3.35 MM Table S.2.3., 3.36 WM Table S.2.1., 3.37 MFM Table S.3.5., 3.38 CDLM Table S.2.5., 3.39 HGM Table S.3.3, 3.40 EVFS Table S.3.3., 3.4.1 NUEMS Table S.2.2. Paragraph: Method of Sealing Category 3 |  |  |  |  |  |  |  |
| 16. Proposal: Use strikeout to show words to be deleted and underline to show new words. (Do not use track changes.) |  |  |  |  |  |  |  |
| 3.30 LMD Table S.2.2, An event logger is required in the device; it must include an event counter (000 to 999), the parameter ID, the date and time of the change, and the new value of the parameter. The event logger information shall may be available at the time of inspection either as a printed copy or in electronic format. provided electronically in lieu of or in addition to a hard copy at the time of inspection, provided the event logger information is retained in the system for future reference. The information may be printed by the device, printed by another on-site device, or transmitted electronically. The event logger shall have a capacity to retain records equal to 10 times the number of sealable parameters in the device, but not more than 1000 records are required. (Note: Does not require 1000 changes to be stored for each parameter.) 3.31 VTM Table S.2.2., 3.32 LPG&AALM Table S.2.2., 3.34 CLM Table S.2.5., 3.35 MM Table S.2.3., 3.36 WM Table S.2.1., 3.37 MFM Table S.3.5., 3.38 CDLM Table S.2.5., 3.39 HGM Table S.3.3, 3.4.1 NUEMS Table S.2.2. An event logger is required in the device; it must include an event counter (000 to 999), the parameter ID, the date and time of the change, and the new value of the parameter. A printed copy of the information must be available on demand through the device or through another on-site device. The information may also be available electronically. The event logger information may be provided electronically in lieu of or in addition to a hard copy at the time of inspection, provided the event logger information is retained in the system for future reference. The event logger shall have a capacity to retain records equal to 10 times the number of sealable parameters in the device, but not more than 1000 records are required. (Note: Does not require 1000 changes to be stored for each parameter.) 3.40 EVFS Table S.3.3., An event logger is required in the device; it must include an event counter (000 to 999), the parameter ID, the date and time of the change, and the new value of the parameter. The event logger information may be provided electronically in lieu of or in addition to a hard copy at the time of inspection, provided the event logger information is retained in the system for future reference. The event logger shall have a capacity to retain records equal to 10 times the number of sealable parameters in the EVSE device, but not more than 1000 records are required. (Note: Does not require 1000 changes to be stored for each parameter.) |  |  |  |  |  |  |  |
| 17. For Handbook 44 proposals, indicate one of the following: |  |  |  |  |  |  |  |
| ☐ Retroactive (Enforceable with respect to all devices) ☒ Nonretroactive (Enforceable on or after the effective date for devices a) manufactured within a state after the effective date, b) both new and used equipment brought into a state after the effective date, c) used in noncommercial applications which are placed into commercial use after the effective date, and d) undergoing type evaluation including devices that have been modified to the extent that a new NTEP Certificate of Conformance is required.) |  |  |  |  |  |  |  |
| 18. Justification: Include national importance, background on the issue, and reference to supporting data or documents. |  |  |  |  |  |  |  |
| The Method of Sealing Categories was initially adopted into Handbook 44 thirty years ago. The use of the Category 3 sealing method has become established, and the method of evaluations has changed from paper records to digital file retention. Technology has advanced in all measurement areas with the integration of electronics for measuring devices utilizing wired and wireless communication transmission. It is not practical to have direct wired connections to measuring devices where there are multiple devices or where access is limited for safety or installation requirements. The proposed language enables non-wired transmission from the measuring device to another device from which information can be accessed electronically and presents printing the event logger information as an option. Over the years, changes have been made to the Category 3 requirements of different codes but have not been made consistently across all the Measuring Codes. It was shortsighted to not address uniformity with the Method of Sealing when the 3.40 EVFS Table S.3.3. and 3.30 LMD Table S.2.2 changes were made Methods of Sealing Category 3 back in 2021 and 2022. The Measuring Sector reviewed the Method of Sealing Category 3 proposal submitted by Michael Keilty of Endress+Hauser and Bill Weakley of Metron. The Measuring Sector found that the proposal did not go far enough to harmonize the language. The Sector recommends taking the language from the 3.40 EVFS Table S.3.3. Method of Sealing Category 3 and change the word “EVSE” to “device” in the last sentence of the requirement. |  |  |  |  |  |  |  |
| 19. Possible Opposing Argument’s: Demonstrate that you are aware and have considered possible opposition. |  |  |  |  |  |  |  |
| Do not change the other code sections Method of Sealing Category 3 because the LMD code section was modified specifically for RMFDs and EVFS because they would be burdened if required to produce a printed copy of the information from the event logger. The proposal allows the event logger information to be provided only electronically. Category 3 Measuring Devices are not secured by a physical seal and could be fraudulently adjusted. Weighing devices in Code Sections 2.XX and MDMD Code Section 5.58 are not recognized in the proposal. |  |  |  |  |  |  |  |
| 20. Requested Action if Considered for NCWM Agenda: |  |  |  |  |  |  |  |
| X Voting Item Developing Item Informational Item Other (Please Describe): |  |  |  |  |  |  |  |
| 21. List of Attachments: |  |  |  |  |  |  |  |